Standard Operating Procedure (SOP)

ESP32 Motion Control System with IMU Logging and Encoder Feedback for Code File controlCode_PCB.py

1. Purpose

This SOP defines the procedures for operating the ESP32-based motion control system the uses the Adafruit ESP32 Feather V2 Part number 5400. The system executes motion trajectories, records IMU (Inertial Measurement Unit) data, and tracks encoder feedback for analysis.

Addinal info: The other code file (controlCode_NoPCB.py) is the same EXCEPT for the pin allocations and there are no encoder inputs due to the overwhelming amount of wiring.

2. System Overview

The system performs the following functions:

- Executes preloaded motion trajectories (X, Y linear axes and A rotational axis)

- Logs IMU data from two sensors over I2C

- Tracks encoder counts for up to six quadrature encoders

- Supports trajectory queuing and looped execution

- Allows manual control via commands (local or network)

3. Hardware Requirements

- ESP32 microcontroller (Adafruit ESP32 Feather V2)

- Motor drivers connected to PWM outputs

- Two IMUs (I2C addresses 0x6A and 0x6B)

- Up to six quadrature encoders (A/B channels)

- External power supply for motors and logic

4. Software Components

- controlCode_NoPCB.py (main control firmware)

- Socket-based command interface (optional GUI or client)

- CSV logging system (imu_log.csv)

5. Initialization Procedure

- Power on the ESP32 system

- Ensure all hardware connections are secure

- Initialize IMUs:

  - IMUs are configured automatically at startup

  - Verify initialization messages in console output

- Start background control loop: background_loop()

- Initialize logging: init_log()

6. Loading Trajectories

- Prepare a trajectory file in CSV format: vx, vy, va, 0.5, 0.5, 0.1
Note: postionToEncoderSteps.py will automatically format the file

- Load trajectory: load_trajectory("filename.csv")

Note: You will need to manually upload the file to the microcontroller. Used Thonny, but any model that is compatible with the ESP32 microcontroller and Micropython will work.

- Verify queue: get_queue_status()

7. Running the System

- Start Execution: start(): Begins the loaded trajectories from the beginning.

- Pause Execution: pause(): Pauses trajectory and allows for the trajectory to resume where it is left off.

- Resume Execution: resume() : Resumes after a pause.

- Stop Execution: stop(): Hard stops the loaded trajectory

  - Stopping will:

    - Halt all motors

    - Reset trajectory indices

    - Flush log buffer to file

8. Looping Trajectories

Set loop count: set_loop_count(n)

- n = 1 → runs once (no loop)

- n > 1 → repeats sequence

- Loop progress tracked internally

9. Manual Axis Control

Move linear axes [mm]:

move_axis('x', 1, 10)   # move +10 mm move_axis('y', 0, 5)    # move -5 mm
Move rotational axis [degrees]: move_axis('a', 1, 90)   # rotate +90°

10. Encoder Monitoring

Retrieve encoder counts: get_encoders()

11. Data Logging

Log file: imu_log.csv

Data recorded:

  - Timestamp

  - Step index

  - Commanded velocities (vx, vy, va)

  - IMU data (acceleration and gyro)

  - Encoder counts (if enabled)

Buffer behavior:

- Data stored in memory buffer

- Written every 25 samples

- Flushed on stop

12. Trajectory Queue Management

Remove trajectory: remove_trajectory(index)
Reorder trajectory: move_trajectory(old_index, new_index)
Clear all trajectories: clear_trajectories()

13. Timing and Performance

- Control loop runs at 100 Hz (DT = 0.01s)

- Encoder updates occur every loop cycle

- Logging occurs every cycle with buffered writes

Limitations:

- High-speed encoders may exceed polling capability

- Consider hardware PCNT for high-frequency signals

14. Safety Considerations

- Always verify direction pin before motion

- Ensure motors are clear of obstructions

- Do not exceed defined speed limits

- Stop system immediately if abnormal motion occurs

Emergency stop: stop() then disconnect power

15. Troubleshooting

| Issue | Possible Cause | Solution |
| --- | --- | --- |
| No motion | PWM not set | Check set_pwm() |
| IMU not responding | I2C wiring | Verify SDA/SCL pins |
| Encoder not counting | Wiring error | Check A/B channels |
| System freezes | Infinite loop or blocking call | Restart ESP32 |
| Missing log data | Buffer not flushed | Call flush_log() |

16. Shutdown Procedure

- Stop system: stop()

- Ensure logs are flushed: flush_log()

- Power down hardware.

17. Future Improvements

- Upgrade encoders to hardware PCNT

- Add real-time GUI plotting

- Implement closed-loop control

- Add fault detection and recovery