Standard Operating Procedure (SOP)

Hardware & Electronics – ESP32 Motion Control System

1. Purpose

This SOP defines the proper setup, operation, and safety procedures for the hardware and electronics of the motion control system, including power distribution, motor control, sensing, and signal conditioning.

2. System Overview

The hardware system is designed to:

- Provide stable multi-rail power distribution

- Drive three stepper motor axes (X, Y, A)

- Acquire encoder feedback from six motors

- Collect IMU (Inertial Measurement Unit) data via I2C

- Interface safely between low-voltage logic and high-power motor drivers

3. Major Components

Power System

- VMS-350C-36 (x3): Primary 36V supply for motor drivers

- 15W 36VDC24V-DC Converters (x3): Step down voltage for the Motor driver logic Microcontroller and peripherals.

Control System

- Adafruit ESP32 Feather V2

  - Main controller

  - Generates PWM (Pluse with modulation) and control signals

  - Handles IMU + encoder processing

- Custom PCB

  - Distributes regulated power

  - Interfaces encoders

  - Contains voltage dividers and signal routing

Motion System

- DM456AI (x3)

  - Drives stepper motors

  - Accepts pulse + direction inputs

- NEMA23-AMT112S (x6)

  - Provides motion and encoder feedback

- AMT112S

  - Quadrature encoder outputs (A/B)

Signal Conditioning

- Optocoupler relay

  - Electrically isolates direction signals

- PWM level shifting (to ~10V)

  - Converts microcontroller PWM (3.3V) → driver-compatible signals

- Voltage dividers

  - Reduce encoder output voltage to safe MCU (Microcontroller) levels

4. Power Distribution

4.1 Primary Power

- Each VMS-350C-36 supplies 36V DC

- Ensure:

  - Proper grounding

  - Load balancing across supplies

- PCB power is 24 Volts input, and powers the MCU and encoders.

5. Wiring and Connections

5.1 Motor Drivers

Each DM456AI requires:

- Pulse (PWM input) (Pin AIN)

- Direction signal (DIR-)

- Motor Power (36V) (VDC and GND [the one directly next to VDC])

- Driver Power (24V) (OPTO and GND [the one directly next to AIN])

- ENA- needs to be shorted to the 24V GND

- Note: DO NOT CHANGE THE DIP SWITCHS. They should all be off and below is an image of how the Driver should look all wired with the motors disconnected and the Switches turned off.  From left to write the pins are 36-volt, Motors (4 pins) 1 and 2, Dip switches, 24V ground, AIN( PWM), 10v (Ignore), ENA-, DIR-, 24V in.

Connections with Microcontroller:

- PWM → boosted to ~10V

- Direction → routed through optocoupler

- Ground → common with system

5.2 Encoders

Each encoder:

- Outputs A/B quadrature signals

Connections:

- Routed through voltage dividers

- Connected to ESP32 GPIO pins

Ensure:

- Signals do not exceed 3.3V

- Clean ground reference

5.3 Microcontroller

The ESP32 Feather V2:

- Receives encoder inputs

- Outputs PWM + direction signals

- Communicates with IMUs via I2C

5.4 IMU Sensors (Adafruit IMUS part number 4754)

- Connected via I2C (SDA/SCL)

- Use pull-up resistors if required

- Ensure unique addresses (0x6A, 0x6B)

6. Startup Procedure

- Verify all wiring connections

- Ensure no short circuits exist

- Power sequence:

  - Enable DC-DC converters

  - Power ESP32

  - Enable motor power (36V supplies)

- Observe:

  - No abnormal heat

  - No unexpected motor movement

  - Note: Stationary motors are in a “locked state” which will produce heat. Not concerned.

7. Normal Operation

- Motors receive PWM + direction signals

- Encoders provide feedback continuously

- IMUs stream motion data

- Control loop synchronizes all subsystems

8. Shutdown Procedure

- Stop all motion (software command)

- Turn off 36V supplies. Turn off the bottom switch with the PCB first.

9. Safety Considerations

Electrical Safety

- 36V supplies can deliver high current → risk of damage or fire

- Always:

  - Use proper wire gauge

  - Secure connections

Signal Isolation

- Optocouplers protect MCU from:

  - Voltage spikes

  - Ground loops

Thermal Safety

- Monitor:

  - Motor drivers

  - DC-DC converters

- Provide airflow or heatsinking if needed

Motion Safety

- Ensure mechanical clearance

- Avoid operating near when people are adjusting or actively working on the stand.

10. Testing & Verification

Power Checks

- Verify output voltages before connecting loads

Signal Checks

- Use oscilloscope or multimeter:

  - PWM signal integrity

  - Encoder transitions

Functional Tests

- Run motors at low speed first

- Verify encoder counts increase/decrease correctly

Calibration/Alignment

- For Calibration for the movement, there are a few steps; first disconnect the central axis.

- Then you will want to set your measurement device to Zero or in other words a starting/zero point (I used calipers).

- Next, use the Move_axis function to move the axis at a set distance, for example 20 mm.

- Finally, measure the distance between your Zero point and the end point. Take the result and divide it by the amount you wanted to go in this case 20mm.

- Finally take that number and multiple it by the MAX_LINEAR_SPEED_Axis varible and that is your new calibration

- Note that you may need to adjust further through trial and error.

- For Alignment for either the linear axis’s you must first disconnect one side (either is fine use your judgement)

- Then use the move axis function to adjust as needed. For further explanation on the move axis function refer to thetestStandAppCodeSOP

11. Common Failure Modes

| Issue | Cause | Solution |
| --- | --- | --- |
| No motor movement | Missing PWM or power | Check driver inputs |
| Erratic motion | Noise or grounding issue | Improve shielding |
| Encoder miscounts | Voltage mismatch | Verify dividers |
| MCU resets | Power instability | Check DC-DC converters |
| Overheating | Excess current | Reduce load |

12. Maintenance

- Periodically inspect wiring

- Check connectors for looseness

- Verify power supply outputs

- Clean dust from components

13. Recommended Improvements

- Add fuses or circuit protection

- Implement hardware emergency stop

- Modify PCB to include Optocoupler relay for and PWM boosters for better space and cost efficiency,

- Remove tape on bottom of circuitry. This was initially put there for safety concerns related to handling.