Standard Operating Procedure

(SOP)

ESP32 Main Control Server

(main.py) and boot.py

1. Purpose

This SOP defines the operation of the main.py script and boot.py running on the ESP32. This script serves as the central command interface, handling system initialization, exposing control functions, and managing remote communication between the ESP32 and external clients (e.g., GUI).

LOGIN Information:

Network: TestStandControl

Password: esp32control

2. System Overview

The main.py script performs three primary roles:

- System Initialization

  - Initializes IMUs and logging

  - Starts the control loop in a background thread

- Command Interface

  - Exposes motion control functions to REPL and external clients

- Network Server

  - Listens for TCP commands on port 1234

  - Executes commands and returns responses

3. Dependencies

- controlCode_PCB module (core motion + logging logic)

- MicroPython libraries:

  - socket

  - machine

  - time

  - _thread (used indirectly)

4. Startup Procedure

Upon boot, the script automatically performs:

4.1 IMU Initialization

controlCode_PCB.init_imu(controlCode_PCB.IMU1_ADDR)
controlCode_PCB.init_imu(controlCode_PCB.IMU2_ADDR)

4.2 Logging Initialization

controlCode_PCB.init_log()

4.3 Start Control Loop

controlCode_PCB.background_loop()

This loop runs continuously in a separate thread and is required for:

- Motion execution

- Encoder updates

- IMU logging

Expected Output:

IMU + Logging initialized
System ready
Command server running on port 1234

5. REPL Command Exposure

The following functions are exposed for direct use in the MicroPython REPL:

- start()

- stop()

- pause()

- resume()

- move_axis()

- load_trajectory()

- set_pwm()

- background_loop()

This allows manual control without network access.

6. Network Command Interface

The system runs a TCP server:

- IP: 0.0.0.0 (all interfaces)

- Port: 1234

External clients (e.g., GUI) send commands as plain text strings.

7. Supported Commands

Motion Control

| Command | Description |
| --- | --- |
| start | Begin trajectory execution |
| stop | Stop execution and flush logs |
| pause | Pause execution |
| resume | Resume execution |

Manual Movement

move_axis <axis> <direction> <distance>

- axis: x, y, or a

- direction: 1 (positive), 0 (negative)

- distance: mm (x/y) or degrees (a)

Trajectory Management

| Command | Description |
| --- | --- |
| load_trajectory <path> | Load trajectory file |
| clear_trajectories | Remove all trajectories |
| remove_traj <index> | Remove specific trajectory |
| move_traj <old> <new> | Reorder trajectories |

Loop Control

| Command | Description |
| --- | --- |
| set_loop <0/1> | Enable/disable looping |
| set_loop_count <n> | Set number of repetitions |

Status / Debug

| Command | Description |
| --- | --- |
| get_queue | Returns queue state |
| status | Returns run state info |

Example response:

{'running': True, 'paused': False, 'step': 42}

8. Command Processing Flow

- Client connects to socket

- Command string is received

- handle_command() parses input

- Corresponding function is executed

- Response is returned:

  - "OK" on success

  - "ERR: <message>" on failure

Data string for queries

9. Logging Behavior

- Logging is handled in controlCode_PCB

- Data is buffered and written periodically

- On stop, logs are explicitly flushed:

controlCode_PCB.flush_log()

10. Connection Handling

For each client:

- Accept connection

- Receive command

- Execute command

- Send response

- Close connection

This ensures:

- Stateless operation

- Reduced risk of hanging connections

11. Error Handling

| Scenario | Behavior |
| --- | --- |
| Invalid command | Returns "UNKNOWN" |
| Exception during execution | Returns "ERR: <message>" |
| Connection issue | Logs error to console |

12. Safety Considerations

- Motors are stopped when stop() is called

- Logging is preserved via flush_log()

- No authentication → system is open to network commands

- Ensure controlled network environment

13. Shutdown Procedure

- Send stop command: stop

- Verify logs are flushed

- Power down ESP32 safely: Meaning switch off the main power. Located at the switches leading into the power supplies.

14. Known Limitations

- Single-client connection handling

- No encryption or authentication

- Blocking socket implementation

- No command queueing (one command per connection)

15. Recommended Improvements

- Add multi-client support

- Implement authentication

- Add non-blocking or async sockets

- Add heartbeat/connection status

- Integrate encoder reset and telemetry commands

16. Troubleshooting

| Issue | Cause | Solution |
| --- | --- | --- |
| No response | Server not running | Restart ESP32 |
| Commands fail | Syntax error | Check format |
| Logs missing | Not flushed | Ensure stop() used |
| IMU not working | I2C issue | Check wiring |
| GUI not connecting | IP/port mismatch | Verify settings |