Standard Operating Procedure (SOP)

Trajectory Generation and Conversion Tool (Python Script)

postionToEncoderSteps.py

1. Purpose

This SOP defines the operation of the Python-based trajectory generation script used to convert position-based motion data into time-stepped velocity commands. The script processes CSV input data and generates a binary motion file compatible with the ESP32 motion control system.

The output file contains synchronized linear and angular velocity commands, including smooth acceleration and deceleration profiles, for execution by the embedded control firmware.

Notes: For example, in CSV format go to 16. Example Image

2. System Overview

The script performs the following functions:

- Loads position data (X, Y, Z, and angular position) from a CSV file

- Interpolates position data as a function of angle

- Generates smooth motion using S-curve velocity profiles

- Computes velocities (vx, vy, vz, va) at fixed time steps

- Creates forward and return motion sequences

- Exports the motion data as a compact binary file

This tool bridges the gap between experimental/simulation data and real-time motor control execution.

3. Software Requirements

- Python 3.14

- Required libraries:

  - tkinter (file selection GUI)

  - pandas (CSV handling)

  - numpy (numerical processing)

  - struct (binary file formatting)

4. Input Data Requirements

4.1 CSV File Format

The input CSV must contain the following columns:

- "Angle" → Angular position (degrees)

- "X (U1)" → X position

- "Y (U2)" → Y position

- "Z (U3)" → Z position

4.2 Data Characteristics

- Angle values must be ordered (monotonic increasing or decreasing)

- Data should be smooth to avoid discontinuities in velocity

- Units should be consistent (typically mm and degrees)

5. Configuration Parameters

The following parameters control motion generation:

| Parameter | Description | Default |
| --- | --- | --- |
| angular_speed | Angular velocity (deg/sec) | 5 |
| DT | Time step (seconds) | 0.01 |
| RAMP_TIME | Acceleration/deceleration time | 0.5 s |
| MAX_PWM | Maximum PWM scaling value | 65535 |
| MAX_LINEAR_SPEED | Max linear velocity (mm/s) | 23.3 |
| MAX_ANGULAR_SPEED | Max angular velocity (deg/s) | 10 |

6. Processing Steps

6.1 File Selection and Loading

- A file dialog prompts the user to select a CSV file

- Data is loaded using pandas.read_csv()

- Position and angle arrays are extracted

6.2 Interpolation Setup

Interpolation functions are created:

- x_func(angle)

- y_func(angle)

- z_func(angle)

These allow continuous position estimation between data points using linear interpolation.

6.3 Motion Profile Generation

An S-curve function is used to smooth acceleration:

𝑠𝑡 = 3𝑡2−2𝑡3

This ensures:

- Gradual acceleration at motion start

- Constant velocity mid-motion

- Smooth deceleration at the end

6.4 Forward Motion Generation

For each time step:

- Compute angular velocity using S-curve scaling

- Update angular position

- Evaluate position at current and next step

- Compute velocities:

  - 𝑉𝑥=𝑥2−𝑥1∆𝑡

  - 𝑉𝑦=𝑦2−𝑦1∆𝑡

  - 𝑉𝑧=𝑧2−𝑧1∆𝑡

d. 𝑉𝑎=   = 𝜔 𝐶h𝑜𝑜𝑠𝑒𝑛 𝑏𝑦 𝑡h𝑒 𝑢𝑠𝑒𝑟

- Determine direction bit:

  - 0 → positive rotation

  - 1 → negative rotation

- Store motion segment

6.5 Return Motion Generation

A reverse trajectory is automatically generated:

- Angular velocity is inverted

- Same S-curve profile is applied

- Motion returns system to starting position

This ensures:

- Cyclic motion capability

- Repeatable experimental conditions

7. Output File Format

7.1 Binary File Structure

Each motion step is stored as:

| Field | Type | Size |
| --- | --- | --- |
| vx | float | 4 bytes |
| vy | float | 4 bytes |
| vz | float | 4 bytes |
| va | float | 4 bytes |
| direction | uint8 | 1 byte |

Total per step: 17 bytes

7.2 File Generation

- User selects output file location via dialog

- Data is written using struct.pack("<ffffB", ...)

- File extension: .bin

8. Output Information

After execution, the script prints:

- Total number of steps

- Total file size in bytes

9. Usage Procedure

- Run the script

- Select input CSV file when prompted

- Wait for processing to complete

- Choose output file location

- Save binary file

- Upload file to ESP32 system

10. Integration with Control System

The generated binary file is intended to be:

- Parsed or converted into trajectory format

- Loaded using load_trajectory() or equivalent

- Executed via the control firmware

Note: If required, an intermediate conversion step may be used to transform binary data into CSV or direct memory structures.

11. Safety Considerations

- Verify motion limits before execution

- Ensure generated velocities do not exceed hardware limits

- Confirm direction logic matches motor configuration

- Test trajectories at low speed before full operation

12. Known Limitations

- Uses linear interpolation (no spline smoothing)

- No real-time visualization of trajectory

- No bounds checking on input data

- Assumes consistent sampling in input dataset

13. Troubleshooting

| Issue | Cause | Solution |
| --- | --- | --- |
| Script crashes on load | Missing columns | Verify CSV headers |
| Erratic motion | Noisy data | Smooth input data |
| Large velocity spikes | Sparse data points | Increase resolution |
| File not usable | Format mismatch | Verify firmware compatibility |

14. Future Improvements

- Add trajectory preview visualization

- Implement spline interpolation

- Add velocity/acceleration limits

- Export directly to firmware-compatible format

- Add GUI for parameter tuning

15. Summary

This script provides a critical preprocessing step in the motion control pipeline, converting positional datasets into smooth, executable motion trajectories. By incorporating S-curve profiles and bidirectional motion, it ensures repeatable, physically realistic movement suitable for experimental validation and system testing.

16. Example Image