Standard Operating Procedure (SOP)

ESP32 Motion Controller GUI (Tkinter Interface)

Code File: testStandApp.py

1. Purpose

This SOP defines how to operate the Python-based graphical user interface (GUI) used to control the ESP32 motion system. The GUI provides remote control of trajectory execution, manual motion, and queue management over a network connection.

Additional Notes: If you want to just test, you only need the .exe version and should not need python installed.

You must connect to the microcontroller wifi with the following information.

LOGIN Information:

Network: TestStandControl

Password: esp32control

For more information on this, navigate to “mainpy_bootpySOP.doc.”

2. System Overview

The GUI application performs the following functions:

- Sends commands to an ESP32 over TCP/IP

- Controls motion execution (start, stop, pause, resume)

- Loads and manages trajectory files

- Provides manual axis movement (X, Y, A)

- Manages a trajectory execution queue

- Displays system status and active trajectory

3. Requirements

Software:

- Python 3.14

- Tkinter (Python Libary)

Hardware:

- ESP32 running compatible control firmware

- Network connection (ESP32 accessible via IP)

Network Configuration:

- Default IP: 192.168.4.1

- Port: 1234

4. Startup Procedure

- Ensure the ESP32 is powered on and running the control firmware

- Connect the host computer to the same network as the ESP32. The network is called TestStandControl and will show after the microcontroller has started. You may need to wait a few seconds and refresh your Wi-Fi.

- Run testStandApp.py through Python or just run the executable(exe) by double clicking the icon. Note: The exe is a single file that can be downloaded straight to desktop.

- Confirm GUI window opens with status: Ready
Note: It should show at the bottom under the trajectory queue. For visual head to #20 where an image of the GUI is shown

5. Connection Behavior

- The GUI communicates via TCP sockets

- A 0.3-second timeout is used to prevent freezing

- If the ESP32 is offline, the GUI will display: Offline / Error

6. Main Controls

Start Execution: Begins executing the loaded trajectory queue: Button: Start

Stop Execution: Immediately halts motion and resets execution: Button: Stop

Pause Execution: Temporarily halts motion without resetting position: Button: Pause

Resume Execution: Continues execution after pause: Button: Resume

7. Loop Control

Allows repeated execution of the trajectory queue.

Set Loop Count

- Enter desired number in input field

- Click Set

Behavior:

- 1 → Run once

- >1 → Repeat sequence

- Invalid input → error message displayed

Note: It will run through every trajectory listed, so only have the ones you want looped to be loaded. You can use the buttons below the queue to help manage the loaded trajectories.

8. Manual Axis Control

Move Linear or Rotational Axes

Inputs:

- Axis: x, y, or a (angle)

- Direction: Positive / Negative

- Distance:

  - mm for X/Y

  - degrees for A

Procedure:

- Select axis

- Select direction

- Enter distance: Should be referenced on the stand

- Click Move

9. Rotate Axis (A Axis Only)

Dedicated control for rotational movement.

Inputs:

- Direction (Positive/Negative)

- Angle (degrees)

Procedure:

- Enter angle

- Select direction

- Click Rotate

10. Loading Trajectories

Steps:

- Click Browse to select a file

- File path appears in input field

- Click Load

Result:

- File is sent to ESP32

- Added to trajectory queue

- Queue display updates

11. Queue Manager

Displays all loaded trajectories and current execution state.

Features:

Refresh: Updates queue display from ESP32

Remove: Deletes selected trajectory from queue

Reorder

- Up → move selected trajectory earlier in line

- Down → move selected trajectory later in line

Clear: Removes all trajectories from queue

12. Queue Display

Each entry shows: [index]: filename

Active trajectory is marked: <-- ACTIVE

13. Status Display

The bottom label shows:

- Last command sent

- Response from ESP32

- Error messages

Examples:

start → OK
Offline / Error
Invalid distance

14. Auto Refresh Behavior

- Queue automatically updates every 2 seconds

- Uses non-blocking network calls

- Prevents GUI freezing when ESP32 is offline

15. Error Handling

| Issue | Cause | Behavior |
| --- | --- | --- |
| ESP32 offline | No connection | "Offline / Error" |
| Invalid input | Non-numeric value | Error message shown |
| Queue unavailable | Network failure | No update |
| Timeout | Slow/no response | Skipped update |

16. Shutdown Procedure

- Click "Stop"

- Close GUI window

- Power down ESP32/ Turn off entire system.

17. Safety Considerations

- Always verify correct axis and direction before movement

- Ensure workspace is clear before starting motion

- Use Stop immediately if unexpected behavior occurs

- Do not rely on GUI alone for emergency stop in critical systems. The main power switches will be a good backup plan and are easily accessible.

18. Known Limitations

- No real-time feedback visualization (IMU/encoders)

- Dependent on network reliability

- No authentication / security on the command interface

- Blocking socket calls still briefly affect responsiveness

19. Recommended Improvements

- Add live telemetry plotting (IMU + encoders)

- Add connection status indicator

- Add configurable IP/port settings

- Implement background threading for networking

- Add trajectory preview visualization

20. Example Image